# Network Configuration and Testing Report – Portugal-Spain DNS and Web Connectivity

## 1. Overview

This report provides a detailed configuration and verification of the Portugal–Spain network setup. The network connects two LANs through a WAN, where Portugal hosts the DNS and Web servers, and Spain contains client PCs. The goal is to enable web access through DNS while restricting ICMP traffic via ACL.

## 2. Network Topology

• Portugal LAN: 172.16.10.0/26
• Spain LAN: 172.16.20.0/27
• WAN Link: 200.10.10.0/30
• Portugal Gateway: 172.16.10.1
• Spain Gateway: 172.16.20.1

## 3. Router Configurations

### Router_Portugal Configuration

| Command | Description |
| --- | --- |
| hostname Router_Portugal | Set hostname |
| interface g0/0/0 | Enter LAN interface |
| ip address 172.16.10.1 255.255.255.192 | Assign IP address |
| no shutdown | Enable interface |
| interface g0/0/1 | Enter WAN interface |
| ip address 200.10.10.1 255.255.255.252 | Assign WAN IP |
| no shutdown | Enable interface |
| ip route 172.16.20.0 255.255.255.224 200.10.10.2 | Static route to Spain LAN |
| access-list 110 deny icmp 172.16.20.0 0.0.0.31 172.16.10.0 0.0.0.63 | Block ICMP from Spain |
| access-list 110 permit tcp 172.16.20.0 0.0.0.31 host 172.16.10.100 eq 80 | Allow HTTP to web server |
| access-list 110 permit udp 172.16.20.0 0.0.0.31 host 172.16.10.53 eq 53 | Allow DNS requests |
| access-list 110 permit ip any any | Allow all remaining traffic |
| interface g0/0/1 | Enter WAN interface again |
| ip access-group 110 in | Apply ACL inbound |

### Router_Spain Configuration

| Command | Description |
| --- | --- |
| hostname Router_Spain | Set hostname |
| interface g0/0/0 | Enter LAN interface |
| ip address 172.16.20.1 255.255.255.224 | Assign LAN IP |
| no shutdown | Enable interface |
| interface g0/0/1 | Enter WAN interface |
| ip address 200.10.10.2 255.255.255.252 | Assign WAN IP |
| no shutdown | Enable interface |
| ip route 172.16.10.0 255.255.255.192 200.10.10.1 | Static route to Portugal LAN |

## 4. Server Configurations

| Device | IP Configuration | Service Configuration |
| --- | --- | --- |
| DNS_Server_PT | IP: 172.16.10.53 /26 GW: 172.16.10.1 | DNS ON A Record: mydns.pt → 172.16.10.100 |
| Web_Server_PT | IP: 172.16.10.100 /26 GW: 172.16.10.1 | HTTP ON Index.html page configured |

## 5. PC Configurations

### Portugal PCs

| PC | IP Address | Gateway/DNS |
| --- | --- | --- |
| PC_PT1 | 172.16.10.10 /26 | GW: 172.16.10.1 ¦ DNS: 172.16.10.53 |
| PC_PT2 | 172.16.10.11 /26 | GW: 172.16.10.1 ¦ DNS: 172.16.10.53 |
| PC_PT3 | 172.16.10.12 /26 | GW: 172.16.10.1 ¦ DNS: 172.16.10.53 |

### Spain PCs

| PC | IP Address (DHCP) | Gateway/DNS |
| --- | --- | --- |
| PC_ES1 | Auto from DHCP (172.16.20.0/27) | GW: 172.16.20.1 ¦ DNS: 172.16.10.53 |
| PC_ES2 | Auto from DHCP (172.16.20.0/27) | GW: 172.16.20.1 ¦ DNS: 172.16.10.53 |
| PC_ES3 | Auto from DHCP (172.16.20.0/27) | GW: 172.16.20.1 ¦ DNS: 172.16.10.53 |

## 6. Verification and Testing Commands

Router_Portugal# ping 172.16.10.100 — Verify local web server connectivity.

Router_Spain# ping 200.10.10.1 — Check WAN link.

Router_Spain# ping 172.16.10.100 — Test remote connectivity through static routes.

PC_ES1> nslookup mydns.pt — Verify DNS resolution → should resolve to 172.16.10.100.

PC_ES1> ping mydns.pt — Expected: Request timed out (ICMP blocked by ACL).

PC_ES1> http://mydns.pt — Expected: Web page loads successfully.