# Implementation of Site-to-Site IPsec VPN in Cisco Packet Tracer

## 1. Objective

To design and implement a Site-to-Site IPsec VPN between two remote LAN networks using Cisco Packet Tracer. The VPN ensures secure, encrypted communication between LAN-A (192.168.0.0/24) and LAN-B (172.16.0.0/24) through the WAN connecting routers R1 and R5.

## 2. Theory

An IPsec VPN (Internet Protocol Security Virtual Private Network) provides encrypted data transmission over insecure networks such as the Internet. In a Site-to-Site VPN, two gateway routers establish an encrypted tunnel to securely connect two LANs. IPsec operates in two phases: Phase 1 (ISAKMP/IKE) establishes a secure channel, and Phase 2 (IPsec) encrypts the actual data traffic.

## 3. Network Topology and IP Addressing

The topology includes five routers (R1–R5), two switches (SW1, SW2), and four PCs. Routers R1 and R5 function as VPN gateways connecting LAN-A and LAN-B.

| Device / Interface | IP Address | Subnet Mask | Description |
| --- | --- | --- | --- |
| R1 G0/0/0 | 192.168.0.1 | 255.255.255.0 | LAN to SW1 |
| R1 G0/0/1 | 200.0.0.1 | 255.255.255.252 | WAN to R2 |
| R2 G0/0/0 | 200.0.0.2 | 255.255.255.252 | WAN to R1 |
| R2 G0/0/1 | 200.0.0.5 | 255.255.255.252 | WAN to R3 |
| R3 G0/0/0 | 200.0.0.6 | 255.255.255.252 | WAN to R2 |
| R3 G0/0/1 | 200.0.0.9 | 255.255.255.252 | WAN to R4 |
| R4 G0/0/0 | 200.0.0.10 | 255.255.255.252 | WAN to R3 |
| R4 G0/0/1 | 200.0.0.13 | 255.255.255.252 | WAN to R5 |
| R5 G0/0/0 | 200.0.0.14 | 255.255.255.252 | WAN to R4 |
| R5 G0/0/1 | 172.16.0.1 | 255.255.255.0 | LAN to SW2 |
| PC-A | 192.168.0.10 | 255.255.255.0 | Host on LAN-A |
| PC-D | 172.16.0.10 | 255.255.255.0 | Host on LAN-B |

## 4. Device Configurations

The following tables contain the final verified configuration for each network device.

### R1

hostname R1 interface GigabitEthernet0/0/0 ip address 192.168.0.1 255.255.255.0 no shutdown interface GigabitEthernet0/0/1 ip address 200.0.0.1 255.255.255.252 no shutdown ip route 172.16.0.0 255.255.255.0 200.0.0.2 access-list 110 permit ip 192.168.0.0 0.0.0.255 172.16.0.0 0.0.0.255 crypto isakmp policy 10 encr aes hash sha authentication pre-share group 2 lifetime 86400 exit crypto isakmp key Coimbra address 200.0.0.14 crypto ipsec transform-set MYSET esp-aes esp-sha-hmac crypto map VPN-MAP 10 ipsec-isakmp set peer 200.0.0.14 set transform-set MYSET match address 110 exit interface GigabitEthernet0/0/1 crypto map VPN-MAP end wr

### R2

hostname R2 interface GigabitEthernet0/0/0 ip address 200.0.0.2 255.255.255.252 no shutdown interface GigabitEthernet0/0/1 ip address 200.0.0.5 255.255.255.252 no shutdown ip route 192.168.0.0 255.255.255.0 200.0.0.1 ip route 172.16.0.0 255.255.255.0 200.0.0.6 end wr

### R3

hostname R3 interface GigabitEthernet0/0/0 ip address 200.0.0.6 255.255.255.252 no shutdown interface GigabitEthernet0/0/1 ip address 200.0.0.9 255.255.255.252 no shutdown ip route 192.168.0.0 255.255.255.0 200.0.0.5 ip route 172.16.0.0 255.255.255.0 200.0.0.10 end wr

### R4

hostname R4 interface GigabitEthernet0/0/0 ip address 200.0.0.10 255.255.255.252 no shutdown interface GigabitEthernet0/0/1 ip address 200.0.0.13 255.255.255.252 no shutdown ip route 192.168.0.0 255.255.255.0 200.0.0.9 ip route 172.16.0.0 255.255.255.0 200.0.0.14 end wr

### R5

hostname R5 interface GigabitEthernet0/0/0 ip address 200.0.0.14 255.255.255.252 no shutdown interface GigabitEthernet0/0/1 ip address 172.16.0.1 255.255.255.0 no shutdown ip route 192.168.0.0 255.255.255.0 200.0.0.13 access-list 110 permit ip 172.16.0.0 0.0.0.255 192.168.0.0 0.0.0.255 crypto isakmp policy 10 encr aes hash sha authentication pre-share group 2 lifetime 86400 exit crypto isakmp key Coimbra address 200.0.0.1 crypto ipsec transform-set MYSET esp-aes esp-sha-hmac crypto map VPN-MAP 10 ipsec-isakmp set peer 200.0.0.1 set transform-set MYSET match address 110 exit interface GigabitEthernet0/0/0 crypto map VPN-MAP end wr

### 4.6 PC Configuration

| PC | IP Address / Subnet Mask | Default Gateway |
| --- | --- | --- |
| PC-A | 192.168.0.10 / 255.255.255.0 | 192.168.0.1 |
| PC-D | 172.16.0.10 / 255.255.255.0 | 172.16.0.1 |

## 5. Final Testing / Verification Commands

| Command | Purpose |
| --- | --- |
| show ip interface brief | Verify that all interfaces are up and assigned correct IPs. |
| ping 200.0.0.14 | Check WAN connectivity between R1 and R5. |
| ping 172.16.0.10 | Verify LAN-to-LAN communication via VPN tunnel. |
| show crypto isakmp sa | Confirm ISAKMP (Phase 1) tunnel establishment — should show QM_IDLE. |
| show crypto ipsec sa | Verify IPsec (Phase 2) — ensure packets are encrypted/decrypted. |

## 6. Conclusion

The Site-to-Site IPsec VPN was successfully implemented between R1 and R5. The configurations ensured encrypted data transfer between LAN-A and LAN-B. Verification confirmed full connectivity and encryption across the WAN.